AquaCore Mk1: Assembly Guide

Tools: Phillips screwdriver, hex keys, wire cutters/strippers, soldering iron, isopropyl alcohol, cloth.

### Preparation

- Clean all metal contact surfaces (both heatsinks, Peltiers) with isopropyl alcohol. Let dry.

### Building the Thermal Stack

- Apply a thin layer of thermal paste to the base of the Hot Side Heatsink (100mm tall, 40mm wide, horizontal fins).

- Place the first Peltier module (40x40mm) onto the paste.

- Apply paste to the top of the first Peltier.

- Place the second Peltier module directly on top of the first, aligning perfectly.

- Apply paste to the top of the second Peltier.

- Seat the Cold Side Heatsink (100mm tall, 40mm wide, vertical fins) on top of the stack.

### Mechanical Assembly

- Secure the entire stack (Hot Sink + 2x Peltiers + Cold Sink) using long screws, nuts, and washers.

- Tighten the nuts evenly in a cross pattern until snug. Do not over-tighten to avoid cracking the ceramic Peltiers.

### Electrical Wiring

- Connect the two Peltier modules in SERIES:

  - Connect the POSITIVE (+) wire from the power supply to the POSITIVE (+) lead of the first Peltier.

  - Connect the NEGATIVE (-) lead of the first Peltier to the POSITIVE (+) lead of the second Peltier.

  - Connect the NEGATIVE (-) lead of the second Peltier to the NEGATIVE (-) wire from the power supply.

- Insulate all wire connections with electrical tape or heat shrink tubing.

### Insulation & Final Setup

- Wrap the sides and back of the Cold Side Heatsink (vertical fins) with closed-cell foam insulation. Leave the vertical fin faces completely exposed to air.

- Place the collection cup directly underneath the exposed vertical fins of the cold-side heatsink.

- Position the unit in a stable, well-ventilated location.

- Connect the 12V, 10A power supply and switch it on.

### Initial Test & Validation

- The hot side heatsink should become warm, and its fan (if attached) should spin.

- The cold side heatsink's vertical fins should become noticeably cold to the touch within 60 seconds.

- In a humid environment, visible condensation (water droplets) should form on the cold vertical fins within a few minutes and drip into the collection cup.